UNIVERSIDAD AUTÓNOMA METROPOLITANA UNIDAD AZCAPOTZALCO

Division: Basic Sciences and Engineering

Technical Report 001:

Initial Hardware Characterization and Low-Level Interface Validation of an Unknown IMU Sensor Using ESP32

Author: Zaldivar Peñaflor Eric Zuriel – 2183082002


Assignment:	 Microelectronics

Professor: 	JOSE LUIS ZAMORANO FLORES




Date: 22/06/2026

Abstract

This technical report documents the initial hardware characterization phase of a TinyML-oriented embedded systems project based on an ESP32 microcontroller and a commercially available inertial measurement unit (IMU) sold as an MPU-6050 module.

The objective of this phase was to establish reliable communication between the ESP32 and the IMU, validate low-level register access, identify the actual device through direct interrogation, and obtain preliminary acceleration measurements suitable for future machine learning dataset generation.

Unexpectedly, the sensor did not identify itself as an MPU-6050. Direct register inspection revealed a WHO_AM_I value of 0x72 instead of the expected 0x68. Despite this discrepancy, successful communication, register manipulation, and accelerometer data acquisition were achieved.

This report documents the methodology, observations, and conclusions obtained during the hardware validation stage.

1. Introduction

TinyML applications rely heavily on the quality and reliability of sensor data. Before constructing datasets or training machine learning models, it is necessary to validate the complete sensing pipeline, including:

- Physical hardware

- Electrical connections

- Communication interfaces

- Sensor initialization

- Data acquisition

The present report focuses exclusively on the hardware validation and low-level communication stage.

2. Hardware Configuration

2.1 Microcontroller

Device:

ESP32 DOIT DevKit V1

Relevant specifications:

- Dual-core Xtensa processor

- 240 MHz clock frequency

- Integrated Wi-Fi

- Integrated Bluetooth

- USB-UART bridge (CP2102)

2.2 Inertial Sensor Module

Commercial label:

MPU-6050

Available pins:

- VCC

- GND

- SDA

- SCL

- XDA

- XCL

- ADO

- INT

Communication protocol:

I2C

3. Electrical Connections

The following wiring configuration was used:

ESP32 → IMU

- 3V3 → VCC

- GND → GND

- GPIO21 → SDA

- GPIO22 → SCL

No additional pins were required during the validation phase.

4. Experimental Procedure

4.1 I2C Bus Validation

An I2C address scan was performed to verify device presence on the communication bus.

Result:

Device detected at address:

0x68

Conclusion:

The sensor successfully responded to I2C discovery requests.

4.2 Device Identification

The WHO_AM_I register was queried.

Register:

0x75

Observed value:

0x72

Expected MPU-6050 value:

0x68

Conclusion:

The device does not identify itself as a genuine MPU-6050.

The sensor is likely a pin-compatible replacement, clone, or a newer member of the InvenSense/TDK inertial sensor family.

4.3 Power Management Inspection

The power management register was examined.

Register:

0x6B

Initial value:

0x40

Interpretation:

Sensor was initially operating in a sleep or low-power state.

4.4 Sensor Activation

A value of 0x00 was written to register 0x6B.

Subsequent read:

0x00

Conclusion:

Bidirectional register communication was verified.

The sensor successfully transitioned to an active state.

4.5 Accelerometer Data Acquisition

Raw acceleration registers were read directly.

Registers:

0x3B – 0x40

Variables obtained:

- AX

- AY

- AZ

Representative measurements:

AX = -1876

AY = 524

AZ = 17556

Additional samples confirmed stable operation and repeatability.

5. Preliminary Observations

The sensor exhibited measurable output noise while stationary.

Approximate observed ranges:

AX ≈ -1900 ± 100

AY ≈ 500 ± 70

AZ ≈ 17600 ± 150

The observed fluctuations are consistent with expected MEMS sensor behavior and environmental disturbances.

No evidence of communication instability was observed.

6. Key Findings

The following capabilities were successfully demonstrated:

- I2C communication

- Register reading

- Register writing

- Sensor wake-up procedure

- Accelerometer acquisition

- Stable serial data transmission

Additionally, the investigation revealed that the installed device does not match the identity of a standard MPU-6050 despite sharing its communication address and register layout.

7. Implications for TinyML Development

The successful acquisition of raw acceleration measurements establishes the foundation required for future stages of the project:

- Sensor calibration

- Noise characterization

- Signal filtering

- Dataset generation

- Feature extraction

- TinyML model training

- On-device inference

At this stage, the hardware platform can already be used for controlled motion data collection.

8. Future Work

Immediate next steps include:

- Identification of the exact IMU model

- Gyroscope register characterization

- Scale factor determination

- Offset calibration

- Noise analysis

- Implementation of digital filtering

- Dataset collection framework

Conclusion

Despite the unexpected identification result obtained from the WHO_AM_I register, the inertial sensor was successfully integrated with the ESP32 platform.

Low-level communication, configuration, and accelerometer acquisition were validated without the use of vendor libraries.

These results confirm that the sensing subsystem is operational and suitable for the next phase of TinyML-oriented experimentation and dataset generation.